Recording a Digital Movie

You will need: 

· Digital video camera

· A charged battery or the power cord

· A mini digital video cassette (such as a JVC miniDV 60 minute cassette)

To begin:

1. If using the power cord, plug it in to the slot labeled DC on the front right panel of the camera.

2. Select TAPE on the dial above the Power light indicator.

3. Insert the cassette tape by sliding the open/eject button and pushing open plastic cover (the metal “jaws” will open by themselves).

4. Click the metal “jaws” together but do not push them into the camera.  They will retract automatically.

5. Close the plastic cover.

6. Turn the selection dial on the back of the camera from the OFF position to the CAMERA position. 

7. To use the LCD screen, press the open button on the left side of the camera and pull out the screen.

8. Remove the lens cap.  ( 

9. Press the silver button to start recording. Press it again to pause.

Using Microsoft Movie Maker

Importing a Video

When finished recording:

1. Turn camera to the OFF position

2. Use the firewire cable (thick, round cable) to connect the camera to the computer

a. Plug small end of cable into DV slot under the dark gray plastic cover on the front of the camera

b. Plug large end of cable into either of the bottom-most ports on the back of the computer tower.

3. Log into the computer.  

4. Turn camera on (selection dial to Camera)

5. On the desktop, go to Programs -> Accessories -> Windows Movie Maker
6. Click on Capture Video in the Movie Tasks menu.  Choose Capture from Video Device.
[image: image1.png]=181]

Fle Edt View Took Cip Play Hep

0w EEE . L |

Collection: Collections
Movie Tasks Diag a clip and diop it o the storyboard below:
1.Capture Video (&

Cepture from video device:
Inport video
Inport pictures
Import auo or music

2. Edit Movie ®

3. Finish Movie ®
Movie Making Tips  (¥)

3 I show Tmeine

o mediato the storyboard to begin making a movie.
<

Ready

@ start| 30 | 5 digta_videa_instructon .. |G} Untitled - Windows M.

TO soamn





7. The “Video Capture Wizard” window will pop up.  Enter a title and a place to save your video file.  Click Next.
[image: image2.png]video Capture Wizard: Canon DV Device

Captured Video File
Enter informaton fo your captured vides fie.

@

1. Enter afile name for your captured video,

Bl (==

Bk ==





8. Choose the default settings by clicking Next.
[image: image3.png]ideo Capture Wizard: Canon DV Device

Video Setting
Select the selfing you want to use ta capture your videa. The capture seting you select
detertines the quaiy and size of the captured video.

@

& Best quali

for playback on my computer (recommended)
Use  you plan to store and edit video on your computer,

C Digital device format (DV-AVI)
Use f you plan to record your final movie back b tape.

 Other settings  [\idex for Packet i (215 g =

Learn mare aboust video setfias.

Setting detals Video fle size

Filetype: Windows Hedia Video
()

Bitrate: 2.1 Mbps

Display size: 640 x 480 pvels
Frames per second: 30

Each minute of video saved with this setting
wil consume 14 B,

Disk space avalbl on diive C: 66.86 GB

B

) start| 8] digal_video_instruction ... |[ 46}, Untitied - Windows M... ] screen_shat - Microsoft.

=lolx|

Type a question for help

- x

850 mam




9. Click on the Start Capture button.  You will see the footage in a preview window.  The capturing process happens in real time, so it may take a while.


[image: image4.wmf] 


10. When you reach the end of the video you wish to capture, click on the Stop Capture button, and then click Finish.

11. Choose Import Video under Capture Video in the Movie Tasks menu.

[image: image5.png]=181]

Fle Edt View Took Cip Play Hep

0w EEE . L |

Collection: Collections
Movie Tasks Diag a clip and diop it o the storyboard below:
1.Capture Video (&

Cepture from video device:
Inport video
Inport pictures
Import auo or music

2. Edit Movie ®

3. Finish Movie ®
Movie Making Tips  (¥)

3 I show Tmeine

o mediato the storyboard to begin making a movie.
<

Ready

@ start| 30 | 5 digta_videa_instructon .. |G} Untitled - Windows M.

TO soamn





12. The “Import File” window will pop up.  Find your file, select it, and click the Import button.  

[image: image6.png]Import options:

2 epeny on bl

=]

QF

21

w o a

Favortes  lessons

File name: rest

Files of type: [Video Files " avi" mpg."mv." mp2. " mp2v."m 7

IV cCreate clips for video files

= npor
=

4

Drag media to the storyboard to begin making a movie.

i

test2 001

=5

s

18 start| 5] digtal_yideo_instruction .. | 1] screen_shot - Microsoft

[ untitied - windows .

(RO 1z




13. You should now see your video in the Movie Maker preview window.

14. You can now unhook the camera from the computer.  (Remember to remove your tape and turn off the camera.)  

15. Drag the movie clip from under Collection: and drop it into the storyboard at the bottom of the screen.

[image: image7.png]=181]
| Fle Edt Vew Toos Cip Py Heb
[0 269 & [[Brats O colections [Beest ()

Collection: test (4) Storgboard: test
Diag a clip and diop it o the storyboard below:

Movie Tasks

Capture Video

Capture from video device
Import video

Import pictures

Import audio o music

Movie Making Tips G

=
DD EIOO

Evdl 5 ® | 2 show Tl

o

| o
Resdy
@ start| 30 | 59} dgital_videa_instruction ... |G}, Untitled - Windows M... 1] Document! - Microsoft 120 szman





Editing Your Movie

Splitting one video into separate clips

Your video will appear as one clip on the storyboard.  To add transitions or effects within the video, pause the playback at the desired point.  On the toolbar menu, choose Clip -> Split.  You can now add a transition between the clips or add an effect to one of the clips.  Repeat as much as needed. 

[image: image8.png]| Fe Edt vew Toos [cp Pay b
EEXIEE #dd o Storyboard] Chl4D

Audo
Movie Tasks video

1. Capture Video

Capture from video device
Import video
Import pictures

St End TrimPaint. CirlsShit+0

Import o or s geg oo ey
2. Edit Movie | combine: it
3. Finish Movi fidge Lot Clyhiftss
= | Rt it

Movie Making Tips |

T@ © | 2 shon Tmelne

=181]

S Start TrimPaine CHlShicE

Clear Trin Paints) - CirlsshiftDel

stoboard below.

1) propertes

Huciges the selected clp to the Ief,

@ start| 30 | 1) digitel_videa_instruction

|[% untitied - Windows ... 1) Document - Mcrosoft

Storyboard: test

2O szomm




Adding video effects

Choose View video effects under Edit Movie in the Movie Tasks menu.  Choose an effect from the list, then drag and drop it onto a video clip in the storyboard.  

Adding transitions

Choose View video transitions under Edit Movie in the Movie Tasks menu.  Choose a transition, then drag and drop it between two video clips on the storyboard.

[image: image9.png]Fle Edt View Took Cip Play Hep

0w Bree Ccoectors [Beomers 5]

Callection: Collections
Movie Tasks Drag a clp and diop it on the stoboard below.

1.Capture Video &)

2. Edit Movie @
View video effects
View videotransitions
Make ttes or credts

3. Finish Movie ®

Movie Making Tips  (¥)

3 I show Tmeine

o mediato the storyboard to begin making a movie.
<

Ready

@ start| 30 | 5 digta_videa_instruction .. |6} ntitled - Windows M.

=181]

2O szemm




Adding titles or credits

Choose Make titles or credits under Edit Movie in the Movie Tasks menu.  Select the type of title or credit, enter the desired text, and click Done, add title to movie (you may have to scroll down to see the Done button.)

[image: image10.png]dows Movie Maker ol
| Fie et vew Took Cip Py Hep forbel - x

[0 H[9- & ([Brets & colections [Beest () 5 1ol x|
Enter Text for Title Fade. In and Out Type a question for help ~ » X
Cick Don' o s th e to the movie 2

Done. add ttle to movie

More options:

OO

EEE

8 ASE Ln3 Coll REC TRK ExT Ow G

@ start| 30 | 59} digital_videa_instruction ... |[{G), untitled - Windows M... 1] Document1 - Microsoft )





Adding audio

1. Switch to timeline view (in the tool bar right above the storyboard). 

[image: image11.jpg]



2. Drag an audio file (from locker, desktop, etc.) into the audio/music bar (under the Video bar in the timeline).  
Burning to a DVD

1. Insert a blank DVD into the DVD drive.

2. Click on Finish Movie in the Movie Tasks menu. 

3. Choose Save to CD.

[image: image12.png]Fie Edt Vew
=

Toos Clp Play Help

=181]

Tasks [ Colections [ Collctions =
Collection: Collections
Drag a clip and dhop it on the storyboard below.

. Capture Video

2. Edit Movie

3. Finish Movie

Movie Making Tips

T show Tinelne:

Drag media to the storyboard to begin making a movi.

) start| [18) untitted - Windows M... _J My Computer | 5 screen_shot - icrosoft .. |

50 %




4. You will be asked to rename the file (this is to keep original video clip separate from the edited version).  Enter titles for both the movie and the DVD.

Confused?

For suggestions, go to Movie Making Tips on the Movie Tasks menu.

[image: image13.png]=181]

Fle Edt View Took Cip Play Hep

0w EEE . L |

Collection: Collections:
Movie Tasks Diag a clip and diop it o the storyboard below:
1. Capture Video ®
2. Edit Movie ®

3. Finish Movie ®
Movie Making Tips (&)

Ho to capture video
How to edt cips

How to add titles, effects, transtions
How ko save and share mavies

3 I show Tmeine

o mediato the storyboard to begin making a movie. |
<

Ready

@ start| 30 | 5 digta_videa_instructon .. |76} Untitled - Windows M... 1] Document1 - Micrasoft

$O soemn





Jennifer Perry

9/06

_1093784545.doc
[image: image1.png]tol x|
Capture Video f
Click the Start Capture button to begin capturing your video. You can capture one or more: i
oo cie g
sops: e =
1 rart Capture:
Start ot m
2 e |
3. Tocapture sncther vdeo i,
Tepat s a2 aga
deo captured: 0:00:00
S of ko 0KB ca
e di
ctmated ok space avaiale
66.85 G remaining onive C: ora-
[V Create clips when wizard finishes
I Mute speakers
(A — = fod]
e
e [ ] cane | pate
K1} I
1=
[T | |
Page 4 Sec 1 44 At tnt Colt

o

) start| 5] digtal_videa_instruction .. | ] screen_shot - Microsoft

T

T80 irasa







